# AI-Controlled Dosing System for pH and EC Regulation prompt

I want to develop a program in PyTorch that will control the dosing of pH and nutrient solutions in a hydroponic system with precision and intelligence.

## System Parameters

The system will have the following input variables:

- Target Parameters:

  - Desired pH value (range: 5.5-6.5)

  - Desired EC value (range: 1.0-3.0 mS/cm)

- Physical Configuration:

  - Tank volume (liters)

  - Peristaltic pump activation times (milliseconds) for each pump

  - Circulation pump flow rate for mixing

  - MOSFET relay control pins for each peristaltic pump

- Solution Properties:

  - pH+ solution concentration (mol/L)

  - pH- solution concentration (mol/L)

  - EC+ solution concentration (mS/cm when undiluted)

  - Multiple nutrient solution options (Solution A and Solution B)

## System Process Flow

- Event-Driven Analysis Cycle:

  - The ESP32 takes samples from the tank at its own frequency

  - Analyzes pH, EC, and other parameters

  - Sends measurement data to PyTorch program whenever new readings are available

  - The baseline properties (initial pH, EC) are automatically determined from first measurement

- Dosing Decision Point:

  - PyTorch model processes measurement data as soon as it's received

  - Calculates precise MOSFET activation durations based on empirical response data

  - Activates appropriate peristaltic pumps via MOSFET relays for calculated durations

- Empirical Learning Approach:

  - Start with minimum pump activation times (e.g., 500ms)

  - Observe actual pH/EC changes resulting from each dose

  - Gradually increase dosing times based on observed responses

  - Build empirical model of pump activation time vs. concentration change

  - Only use theoretical dilution equations when sufficient historical data exists

- Nutrient Solution Transition Detection:

  - Monitor EC depletion rates and patterns over time

  - Calculate and track nutrient consumption slopes

  - Automatically detect when plants are ready to transition from Solution A to Solution B

  - Trigger solution change when rapid EC drops are observed with stable pH (indicating accelerated metabolism)

## System Architecture & Data Flow

- Component Integration:

  - Use a Python launcher script to start both dashboard and PyTorch controller

  - Dashboard handles all direct file I/O operations

  - PyTorch controller focuses on analysis and decision-making

- Inter-Process Communication:

  - PyTorch controller sends dosing decisions to dashboard via ZeroMQ or socket connection

  - Dashboard receives dosing commands and handles both:

    - Writing to CSV files

    - Sending commands to ESP32

  - This prevents file access conflicts and data corruption

- Startup Sequence:

  - Launcher script initializes both components

  - Dashboard establishes serial connection to ESP32

  - PyTorch controller connects to dashboard's command interface

  - System begins normal operation with coordinated data flow

## Physical-Chemical Relationships

- pH relationship: pH = -log10[H+], with [H+][OH-] = 10^-14

- EC dilution follows: ECfinal = (ECsolution × Vsolution + ECwater × Vwater) ÷ (Vsolution + Vwater)

- Temperature compensation for both pH and EC measurements

- Depletion ratio = EC drop rate / time period (indicating plant consumption rate)

- Note: Theoretical equations serve as guidance only; system relies primarily on observed responses

## Machine Learning Integration

- The PyTorch model should learn from:

  - Current error (difference between target and measured values)

  - Direct relationships between pump activation times and resulting concentration changes

  - Rate of change in measured values between sequential readings

  - System response to previous dosing actions

  - Tank volume impact on dosing requirements

  - Historical patterns in pH/EC drift

  - EC depletion rates and pattern changes indicating plant growth stages

- Model should predict:

  - Optimal pump activation time (milliseconds) for each adjustment needed

  - Expected system response after dosing

  - Confidence interval for the prediction

  - When to transition between nutrient solutions

## Data Storage & Logging

- Record in existing dashboard CSV file:

  - Timestamp of each dosing action (using existing ESP32 timestamps)

  - Solution type used (pH+, pH-, or EC+)

  - Pump activation duration (milliseconds)

  - All dosing data added as new columns to the right of existing columns

  - No modification or removal of existing data/columns

- Calculate and store efficiency metrics (time to target, stability)

- Record EC depletion ratios and consumption slopes for growth stage analysis

## Failsafe Mechanisms

- Implement upper limits on dosing quantities based on tank volume

- Use conservative incremental dosing until system response is well understood

- Verify sensor readings are within reasonable ranges

- Emergency stop if rapid, unexpected changes detected

## Calibration & Learning

- Incorporate daily sensor calibration data

- Progressive refinement of dosing models with accumulating data

- Adaptation to changing conditions (seasonal water changes, etc.)

- Learn optimal timing for solution transitions based on historical data

- Continuously improve pump activation time vs. concentration change models

## Future Scalability

- Modular design to incorporate new sensors (nitrate, phosphate, etc.)

- Framework for integrating interference correction between sensor readings

- API for external system integration

Algorithm prompt

# Chemistry-Aware Adaptive Dosing System for Hydroponics

## Problem Statement

The current hydroponics dosing system has a fundamental flaw - it operates without true chemical understanding. A proper science-based dosing system requires chemical principles, accurate calculations, and adaptive learning.

## Key Variables

- pH_minus_molarity = 0.1M KOH solution

- pH_plus_molarity = 0.1M HCl solution

- EC_up_concentration = 200,000 ppm (mg/L)

- Pump Flow Rate = 50 ml/min

- Tank volume = 3L

- minimum_dosing_time = 25ms

## Core Requirements

### 1. Solution Concentration Knowledge

- Store concentration values for pH up/down solutions in mol/L

- Store concentration values for nutrient solutions in ppm

### 2. Volume Calculations

- Pump time to volume conversion: Dosing Volume(mL) = flow_rate(mL/min) × time(ms) × (1/1000) × (1/60)

- Chemical dilution calculation: Dosing Volume(mL) = tank_volume(L) × 1000 × (concentration change / dosing solution concentration)

### 3. Chemistry Mathematics

- pH logarithmic calculations: pH = -log10([H+])

- Dilution formula: C₁V₁ = C₂V₂

- For pH: Calculate mmol of H+/OH- needed for desired pH shift

### 4. Adaptive Learning Approach

- Initial Phase (insufficient machine learning data):

  - Calculate theoretical dose volume

  - Apply 50% safety factor to avoid overshoot

  - Convert to pump time: pump_time_ms = (volume_ml / flow_rate_ml_per_sec) × 1000

- Learning Phase (after each dosing):

  - Measure actual parameter changes

  - Calculate effective flow rate: flow_rate = dosed_volume / observed_change

  - Update model with new correlation data

- Optimization Phase (for next dosing cycle):

  - Recalculate theoretical dosing using updated parameters

  - Incrementally adjust by adding minimum_dosing_time (25ms)

  - Verify against safety limits

This system starts with chemical first principles, then continuously optimizes based on observed behavior, creating a self-improving dosing solution that understands the actual chemistry of the hydroponic system.

### Falta Comprar

| Etapa | Objecto | Provedor | Link |  |
| --- | --- | --- | --- | --- |
| Dosificacao | Solucao A | Leroy Merlin |  |  |
| Prototipagem | Aparafusador/brocas Madeira | Leroy Merlin |  |  |
| Mix | Bomba Aquario | Amazon/Temu | Feito |  |
| Mix | pH Salts | Amazon/Temu | Feito |  |
| NEXT STEP Prototype | PP | Aliexpress | Feito |  |
| NEXT STEP Prototype | pH sensor and PCB | Aliexpress |  |  |
| NEXT STEP Prototype | Nitrate cmbined sensor | Aliexpress |  |  |
| NEXT STEP Prototype | 3 way valves | Aliexpress | Feito |  |
| NEXT STEP Prototype | Regulador de tensao | Aliexpress/Temu | Feito |  |

|  |  | NPK (Mg-S) | Link | Preco |
| --- | --- | --- | --- | --- |
| Hy-Pro Hydro | Solution A | 3-0-2 |  |  |
|  | Solution B | 1-3-4 |  |  |
| Plagron | Solution A | 3-0-1 |  |  |
|  | Solution B | 1-3-6 |  |  |
| FloraDuo | Solution A | 5-0-6 (1 - 0 - 1.2) |  |  |
|  | Solution B | 1-5-4 |  |  |
| Adubo para horta SIRO PROTEAS LIBERT 120GR |  | 16-0-10 (1.5 - 0 - 1) | Link | 2 euros |
| Adubo granulado universal Siro Libert 120gr |  | 18-6-8 (4-9) (3 - 1 - 1.3) | Link | 2 euros |
| Adubo granulado fltoração Siro Libert 120gr |  | 13-13-13 (3-8) (1 - 1 – 1) | Link | 2 euros |
| Adubo granulado orquídeas Siro 120gr |  | 16-6-12 (2-4) (2.6 - 1 – 2) | Link | 2 euros |
| Adubo húmus horta SIRO 1lt | 1 tampa para 5L água | 3-0-3 | Link | 6euros/L |

Novidades

V1

V2 (02-02-2024)

Actuators -  everything ok

V3 (03-02-2024)

Actuators -  everything ok

pH sensors connected

EC sensors connected

Temperature sensor - x

PrgCal V4 - V

Dashboard V4 - V

Nitrate sensor ( Temperature ) – estranho

V4 (11-03-2024)

Implementacao de Calibracao pH sensor ( Scenario 1, Scenario 2, Scenario 3)

Definição de pH values Calibration no cvs (PRG Cal)( pHexcepted Value)

Measure (0,1) (read, dont read)

pHexpected Value (-1,>0) ( dont read, pH calibration Value)

V4p2 (11-03-2024)

X e Y de PRGCal e PRGSample, apenas 1 vez

V4p3 (13-03-2024)

X e Y de PRGCal e PRGSample, repete

Dashboard terminal output optimized

V3 to waist connected to GPIO18, and PGRCal and PRGSample updated

V6

ADS1115s ( 16 bit)

Filter Comparison

Project Components and Variables Checklist

V4p5 (14-03-2024)

|  | Event (x PRGcal + y PRGSample) | Voltage | Calibration | Interpolation (Value) |
| --- | --- | --- | --- | --- |
| Actuator | V | - | - | - |
| Photodiode | V |  |  |  |
| ECsensor | V | V | X | X |
| pHSensor | V | V | V | X |
| Nitrate Sensor |  | V | X | X |
| Events (x and Y) | V |  |  |  |
| Log | V | V | V |  |
| Graph | V | V | V |  |
| Data.cvs | V | V | V |  |

Project Components and Variables Checklist

V4p6 (14-03-2024)

|  | Event (x PRGcal + y PRGSample) | Voltage | Calibration | Interpolation (Value) |
| --- | --- | --- | --- | --- |
| Actuator | V | - | - | - |
| Photodiode | V | - | - | - |
| ECsensor | V | V | X | X |
| pHSensor | V | V | V | V |
| Nitrate Sensor | V | V | X | X |
| Events (x and Y) | V | V | V | V |
| Log | V | V | V | V |
| Graph | V | V | V | V |
| Data.cvs | V | V | V | V |

Project Components and Variables Checklist

V4p8 (14-03-2024)

|  | Event (x PRGcal + y PRGSample) | Voltage | Calibration | Interpolation (Value) |
| --- | --- | --- | --- | --- |
| Actuator | V | - | - | - |
| Photodiode | V | - | - | - |
| ECsensor | V | V | X | X |
| pHSensor | V | V | V | V |
| Nitrate Sensor | V | V | X | X |
| Events (x and Y) | V | V | V | V |
| Log | V | V | V | V |
| Graph | V | V | V | V |
| Data.cvs | V | V | V | V |

V4p9 (14-03-2024)

Hydrodynamic conditions (PRGx 4p7 and PRGx 4p8)

pH meter with 12 bit ADC

| Wait (s) | Potentiometer 1 (Close to BCN connector) | Potentiometer 2 | RSD | (VpH) | Range |
| --- | --- | --- | --- | --- | --- |
| 110 | Min | Max | 4.8 | 6 ± 0.29 | 5.7-6.3 |
| 110 | Min | Min | 4.2 | 6 ± 0.25 | 5.7-6.3 |
| 220 | Min | Min | 6.2 | 6 ± 0.37 | 5.6-6.4 |

vvsd
V6

Filter Comparison

| Filter | Window Size | Std Deviation | pH error |
| --- | --- | --- | --- |
| Median | 50 | 0.00226142 | 0.015 |
| Median | 100 | 0.00248251 | 0.015 |
| Savitzky Golay P2 | 50 | 0.00221321 | 0.015 |
| Savitzky Golay P2 | 100 | 0.00218033 | 0.015 |

Reproducibility

| Filter | Waiting Time | pHvoltage RSD (%) | pH value RSD | Sample time |
| --- | --- | --- | --- | --- |
| Median 60 points | 500 | 0.09 |  | 10 min |
| Median 60 points | 250 | 0.17 |  |  |
| Median 60 points | 300 |  |  | 5 min |
| Median 60 points | 125 | 0.41 |  |  |
| Median 60 points | 50 | 1.34 |  | 2.5 min |

| Filter | Waiting Time | Dependency |  |  |
| --- | --- | --- | --- | --- |
| Injection time | 30 | Fluidics |  |  |
| Air Injection Time | 10 | Fluidics |  |  |
| Wait time |  | Sensors and Filter |  |  |
| Measure time | 1 | ESP32 |  |  |
| Cleaning time | 59 | Fluidics |  |  |

| ADC | Error pH value | Increase of bits compared to previous |  |  |
| --- | --- | --- | --- | --- |
| 12bits | 2.1 | Fluidics |  |  |
| 16bit | 0.2 | 33 % |  |  |
| 24bits | 0.01 ? | 50 % |  |  |

# Soluções

### Recommended NaOH Concentrations for pH Adjustments

- 0.1 M NaOH:
Suitable for controlled pH adjustments in most cases.

- 0.01 M NaOH:
Best for fine-tuning, minimizing overshooting.

- 1 M NaOH:
Too strong; difficult to control small pH changes.

### Preparation of NaOH Solutions

#### To Prepare a 0.1 M NaOH Solution:

- Weigh 4 grams of NaOH pellets.

- Dissolve the pellets in 1 liter of distilled water.

### Preparation of a 0.1 M HCl Solution from Muriatic Acid

#### Desired Concentration and Final Volume:

- Desired concentration: 0.1 mol/L (0.1 M)

- Final volume of solution: 500 mL (0.5 L)

#### Calculations:

- Determine the Number of Moles Needed

  - Formula:
n (mol) = Concentration (mol/L) × Volume (L)

  - Calculation:
n = 0.1 mol/L × 0.5 L = 0.05 mol of HCl

- Calculate the Volume of Commercial Muriatic Acid Needed

  - Commercial muriatic acid (hydrochloric acid) is typically about 37% by mass, which is approximately 12 M (mol/L).

  - Formula:
Volume (L) = n (mol) ÷ Concentration (mol/L)

  - Calculation:
Volume = 0.05 mol ÷ 12 mol/L = 0.00417 L (or 4.17 mL)

### Conclusion

To prepare 500 mL of each solution, use the following summary:

- 0.1 M NaOH: Weigh 2 grams of NaOH pellets and dissolve in 500 mL of distilled water.

- 0.1 M HCl from Muriatic Acid: Measure 4.17 mL of muriatic acid and dilute 500 mL of distilled water.

To do

| ESP32 |  |  |
| --- | --- | --- |
| Tarefa | State | Finish |
| Enviar tempo |  |  |
| Media |  |  |
| bitwise operations (events.h) |  |  |
| Temperatura |  |  |
| Gravar dados para um ficheiro |  | Melhorar |
| PRGCal |  |  |
| PRGSample |  |  |
| REF |  | Verificar |
| Measure |  | Verificar com PH sensor! |
| RCal |  | Verificar com PH sensor! |
| Interpolation |  | Verificar com PH sensor! |
| Real time - delay de 8 s |  | Verificar |
| Separar tudo em ficheiros |  |  |
| Calibrar sensor EC |  | Comprar referencia |
| Calibrar pH |  | Comprar Referencia |
| pH up |  | NaOH/KOH |
| pH down |  | Nitric acid/Phosphoric acid |
| Calibrar ISE nitrato |  | KNO3 |

To create a program where you assign a bit value for the state of each actuator and then program a sequence of events, you can use bitwise operations to manage the states of the actuators. Each bit in a byte (or larger data type) can represent the state of a specific actuator. You can then create an array of these bytes to represent the sequence of events.

ESP32 Code : 			VSCODE:	main.cpp

Initialize ESP32 to UART	VSCODE: 	View-> Command Pallete->Serial Monitor

platformio device monitor --baud 115200

Initialize Dashboard 		TERMINAL:	cd ~/Desktop

python3 dashboardv3.py

Initialize Dashboard view 	CHROME:	 http://127.0.0.1:8050/

| Action | Terminal | Code |
| --- | --- | --- |
| ESP32 Code | VSCODE | main.cpp |
| Initialize ESP32 to UART | VSCODE | View-> Command Pallete->Serial Monitor |
|  | Mac | platformio device monitor --baud 115200 |
| Initialize Dashboard | Mac | python3 "/Users/penutaco/My Drive/Teste LEDs/0 Dasboard/dashboardv3.py" |
|  |  | python3 dashboardv3.py |
| Initialize Dashboard view | CHROME: | http://127.0.0.1:8050/ |

Ser a ordem de iniciacao….talvez seja preciso começar pelo gráfico e depois o platformio

Program Events (Bitwise Operation)

| Action | Terminal | Code |
| --- | --- | --- |
|  | MAC | python3 "/Users/penutaco/My Drive/Teste LEDs/1 PRG Events protocol/2 GenerateEvents.py" |

Experimental Section:

1. pH value repetitively variation for potentiometer setting and waiting time

SW version: 	4p9 Teste LEDs V4p9 (17 03 2025)

pH sensor: 	12bit Ads

Event files: 	PRGcal 	V4p7p1 and V4p8

PRGsample 	V4p7p1 and V4p8

Reagents

|  | pH Value (Potentiometer 1 minimum, Potentiometer 2 maximum, 110 s wait) | pH Value (Potentiometer 1 minimum, Potentiometer 2 minimum, , 110s wait time) | pH Value (Potentiometer 1 minimum, Potentiometer 2 minimum, , 220s wait time) |
| --- | --- | --- | --- |
| pH | 6.93 | 6.87 | 7.56 |
| Error | 0.57 | 0.48 | 0.64 |
| Standard Deviation | 0.335 | 0.288 | 0.466 |
| RSD | 4.83 | 4.18 | 6.16 |

2. pH value repetitively for ADC 12V vs 16V

SW version: 	4p9 Teste LEDs V4p9 (17 03 2025)

pH sensor: 	pH sensor: 	12bit Ads (Potentiometer1: minimum, Potentiometer 2: minimum)

Event files: 	PRGcal 	V4p7p2

PRGsample 	V4p7p2

Calibration Reagents	pH 4.01 Salt (new)

pH 6.89 Salt (new)

Sampling reagents		pH 6.89 Salt (new)